Protection
DIRECTIVE       

10/92 -- P.N. 845
1-4-1-601, p. 49


Appendix 2, p. 4


SMOKE INTRUSION REPORT

Form 1-4-1-301
A.
SMOKE ORIGIN:
           Unit                          District
Legal
Owner



Ign
 Date

        Number(s)                 Forest
Descr
Class
Elev
Acres
Tons
Time
Burned
        Pringle RNA       Deschutes         T-21 R-9 Sec 35   USFS       4500         200
800          1235
10-11-2006


        Pringle RNA       Deschutes         T-21 R-9 Sec 35   USFS       4500         130           650          1145        10-12-2006

B.
INTRUSION DESCRIPTION:
1.
Designated Area Affected       Bend

2.
Date 10/12-13/06   Time 0600   Smoke entered area 0600   Duration __2___hours

3.
Type:  Main Plume _______    Residual Smoke _X             Drift Smoke  

4.
Describe Smoke Behavior (including distances and elevations of base of plume)   During ignition smoke was rising approximately 7,000 to 10,000 ft. according to Round Mt. lookout both days.  There was upslope 20 ft wind and transport winds were ENE for 10-11.  There was WNW 20 ft. wind and transport winds were WNW for 10-12

5.
Narrative (or Comments) of intrusion.   I believe that the intrusion was the result of residual smoke that moved  into Bend during the early morning when the winds shifted from a North wind to a South wind during the early morning hours. 

C.
FORECAST AND INSTRUCTIONS:
1.
Forecast transport wind direction and speed at ignition time and for next 12 hours:   Smoke management forecast and instructions were for Zone 605-611 Morning: mixing height 1000 to 1700 ft. Transport winds light and variable but favors N to E and is controlled by local terrain.  Afternoon:  Mixing height 1700-2700 ft.  Transport wind similar to morning Surface winds increases to NNW to NNE at 4-8 mph.  Evening:  Mixing height lowers to 1000 ft.  Transport winds increases to N to NE at 4-8 mph.  Surface winds becomes light and variable and is controlled by local terrain. 

2.
Observed transport wind direction and speed at ignition time and for next 12 hours.  Transport winds were ENE 10-11 and WNW 10-12 estimate 2-4 mph.

3.
Forecast surface wind direction and speed at ignition time and for next 12 hours (24 hours if residual smoke was a factor) According to the spot weather forecast 20 ft winds NW 2-4 becoming drainage and ridge top winds Northeast 3-7 for 10-11.  NNW 4-6 becoming North 3-6 and ridge top winds Northwest 4-8.

4.
Observed surface wind direction and speed at ignition time and for next 12 (24) hours.  Ignition time NWN  0-4 after 1800 0-2 NE.

5.
Were significant changes in transport or surface wind conditions forecast  ______observed______  Describe any changes that occurred.  None at time of ignition or after ignition was completed.

6.
What were general weather conditions during the burn period (include conditions at least 6 hours after ignition stopped.)  Give sky conditions, type and height of clouds, precipitation etc., be specific.   During the Burn conditions were Temperature 62-71 deg.  37-25% RH  Winds 0-4 NNW to NE

7.
Was Salem consulted about observed weather that was different than forecast? ______________________


_______________________________________________________________________________________

8.
What were Smoke Management Instructions?  Written and/or verbal:___________Both________     
D.
WHAT WERE THE FUEL MOISTURES AT IGNITION TIME:
  
1 hour____8__    10 hour___4.4__    100 hour___11___    1000 hour_11.5______

Protection
DIRECTIVE       

10/92 -- P.N. 845
1-4-1-601, p. 49


Appendix 2, p. 4


SMOKE INTRUSION REPORT

Form 1-4-1-301
E.
OTHER VISIBILITY RESTRICTING SOURCES PRESENT:
  
Field Smoke____   Resident Emissions___   Ag Smoke____   Unable to identify____________________

  
Dust_________ Other prescribed Fire Smoke_______________________________________________

  
Other (Specify)________ Wildfire Smoke (Fire's Name)





            

F. 
EXPLAIN SPECIFICALLY THE CAUSE OF THE INTRUSION.  Has the cause been the result of previous intrusions? In the early morning hours the wind shifted from Northeast to a Southerly flow.
G.
COMMENTS: _________________________________________________________________ 

SECTION H THROUGH L TO BE COMPLETED BY SALEM FORECASTER:

H.
INTRUSION INTENSITY:
1.
Average DA prevailing visibility for 3 hours prior to start of intrusion ___NA_____ miles.

2.
Lowest prevailing visibility during duration of intrusion __NA___ miles.

3.
Average DA nephelometer for 3 hours prior to start of intrusion _____.51_________

4.
Highest nephelometer during duration of intrusion ___________4.48_______________________

5.
Classification based on visibility or nephelometer:

 
Light___   Moderate _X_   Heavy ___   Unknown or can't determine ___ 


No classification (due to other sources) ____

  
If moderate or heavy, the number of hours in those categories:   Moderate__8___  Heavy _______

I.
OBSERVED MIXING DEPTH FROM NEAREST RAOB OR UPPER AIR SITE.  (Identify any shear layers.) __Salem 10/12 = 1514 ft  10/13 = 2381 ft                Medford 10/12 = 2691 ft   10/13 = 3309 ft__________

J.
GENERAL SYNOPTIC CONDITIONS, BOTH LARGE AND SMALL SCALE.  Be as specific  as possible with feature location.  Include surface and upper air map type.  Blocking pattern.  Ridge north cut off low south.  Surface thermal trough along coast, moving inland on the 13th.  MT sfc 2->3, ua 4
K.
WERE  FORECASTS  ADEQUATE (Y/N) _Y__  Why _Poor mixing, nighttime light and variable surface wind highlighted.
L.
WERE INSTRUCTIONS ADEQUATE (Y/N) __N___  Why _Distance from downwind DA could have been greater given stability, L/V nighttime wind and tendency for smoldering units._

M.
COMMENTS

Bend nephelometer shows 7 hours impact on Oct 12 and 10 hours impact on Oct 13.  Flow down Deschutes River during the night has been a historical problem.  Minimizing nighttime smoke would be advantageous.

__Maurice Evans  _______      


     ____Jim Trost___

  
District/Forest Representative              

    Smoke Management Forecaster








____06-04___________________

(10/92)





                       Intrusion No.
